[image: image1.jpg]


RS 152 & RS 153
Perchloroethylene Technical Features
	  
	  
	RS 152
	RS 153

	LOAD CAPACITY
	Lbs 
	35
	35

	BASKET
	
	
	

	Volume
	cu ft
	10.1
	10.1

	Diameter
	inches
	31.5
	31.5

	Depth
	inches
	22
	22

	Wash speed
	rpm
	40
	40

	Extract speed
	rpm
	385
	385

	Door opening
	inches
	19.7
	19.7

	TANKS
	
	
	

	Useful volume tank 1
	gal
	17.9
	21.7

	Useful volume tank 2
	gal
	29.6
	21.7

	Useful volume tank 3
	gal
	-
	35.1

	STILL
	
	
	

	Useful Still Volume at half inspection
	gal
	38
	38.3

	Total Still Volume
	gal
	43.3
	47.5

	NYLON FILTER
	
	
	

	Volume nylon filter housing
	gal
	9.2
	9.2

	Nylon filter area
	ft2
	22.6
	22.6

	Filter disks
	Nr
	14
	14

	CARTRIDGE FILTER
	
	
	

	Volume filter housing
	gal
	3.4
	3.4

	Tubes number
	Nr
	1
	1

	Cartridges quantity
	Nr
	1
	1

	POWER  (220 V – 3 ph – 60 Hz)
	
	
	

	Wash motor
	Hp
	0.65
	0.65

	Extract motor
	Hp
	2.4
	2.4

	Solvent pump motor
	Hp
	0.75
	0.75

	Fan motor
	Hp
	1.5
	1.5

	Recuper  (U2000 models)
	Hp
	1
	1

	Refrigerator compressor
	Hp
	3.25
	3.25

	Nylon filter motor drive
	Hp
	0.75
	0.75

	Idromatic pump  (U2000 models)
	Hp
	0.75
	0.75

	Still heating elements (if full electric)
	Kw
	7.5
	7.5

	Working power (ELECTRIC HEATED)
	Kw
	18
	18

	Working power (STEAM HEATED)
	BHP
	2.5
	2.5

	MACHINE WEIGHTS REGULAR Version
	
	
	

	Empty machine weight
	lbs
	2,712
	2,778

	Machine weight with solvent
	lbs
	3,576
	3,924

	Static charge on the floor with solvent
	lbs/ft2
	158
	151.4

	Dynamic charge on the floor (+ 10%)
	lbs/ft2
	207
	195

	Packaged machine weight
	Lbs
	2,828
	2,949

	MACHINE WEIGHTS U2000 Version
	
	
	

	Empty machine weight
	lbs
	2,712
	2,778

	Machine weight with solvent
	lbs
	3,576
	3,924

	Static charge on the floor with solvent
	lbs/ft2
	158
	151.4

	Dynamic charge on the floor (+ 10%)
	lbs/ft2
	207
	195

	Packaged machine weight
	Lbs
	2,866
	3,616

	MACHINE DIMENSIONS
	
	
	

	(A) Width front Machine
	inches
	39.6 + 4
	65

	(B) Depth
	inches
	84.1
	57.5

	(C) Height with Fan
	inches
	79.1
	78.9


Specifications subject to change without notice. 
